| ## NAME | ## SHASTRI PANCHAM |
| --- | --- |
| ## COURSE | ## DIMPLOMA IN CYBERSECURITY |
| ## SUBJECT | ## HARDWARE |

Assignment: Ethernet and Fiber Optic Cables

1. Three Key Differences Between Ethernet and Fiber Optic Cables

| Feature | Ethernet Cable (Copper) | Fiber Optic Cable |
| --- | --- | --- |
| Transmission medium | Uses copper wires to transmit electrical signals. | Uses thin glass or plastic fibers to transmit light signals. |
| Speed and distance | Commonly supports 1 Gbps or 10 Gbps, depending on cable category and distance. Standard twisted-pair Ethernet channels are generally limited to 100 metres. | Supports very high speeds over longer distances, from hundreds of metres to many kilometres depending on fiber type and equipment. |
| Cost and installation | Generally cheaper and easier to install for short connections. | Usually costs more to install because it may require optical modules and specialized tools. |

Real-world examples:

- Ethernet cable: Connecting a home Wi-Fi router to a desktop computer or connecting gaming PCs in a café.

- Fiber optic cable: Connecting buildings across a college campus or linking servers within a data center.

2. Straight-Through Ethernet Cable Wiring — T568B Standard

A straight-through Ethernet cable has the same wiring standard at both ends. For this assignment, both connectors use the T568B standard.

Pin numbers and wire colours

T568B wiring order — Pin 1 to Pin 8

Pin 1

White/Orange

Pin 2

Orange

Pin 3

White/Green

Pin 4

Blue

Pin 5

White/Blue

Pin 6

Green

Pin 7

White/Brown

Pin 8

Brown

A straight-through cable uses the same pin order at both ends. The eight wires must be arranged correctly to ensure reliable data transmission.

3. Step-by-Step Guide to Attaching an RJ45 Connector to a CAT6 Cable

Watch this tutorial before writing the practical steps:

How to Crimp RJ45 Connector

YouTube tutorial · T568B wiring standard

Watch the tutorial

Tools required:

- CAT6 Ethernet cable

- RJ45 connector compatible with the cable

- RJ45 crimping tool

- Cable stripper or cutter

- LAN cable tester

Procedure:

- Cut the cable: Measure the required length and cut the CAT6 cable cleanly.

- Strip the outer jacket: Remove approximately 2–3 cm of the outer insulation without damaging the inner wires.

- Separate the wires: Carefully untwist and straighten the eight wires.

- Arrange the wires: Place them in T568B order: White/Orange, Orange, White/Green, Blue, White/Blue, Green, White/Brown, Brown.

- Trim the wires: Cut all eight wires evenly so that they fit inside the RJ45 connector.

- Insert the wires: Push them into the connector, ensuring each wire reaches the end and the outer cable jacket extends inside the connector for strain relief.

- Crimp the connector: Insert the connector into the crimping tool and squeeze the handles firmly to secure the contacts.

- Repeat at the other end: Use the same T568B standard on the other end to make a straight-through cable.

- Test the cable: Use a LAN cable tester to check that all eight conductors are connected correctly.

Result: A CAT6 Ethernet cable with RJ45 connectors is prepared and tested for network connectivity.

iFixit Repair Guide

+1

4. Gaming Café Setup: 10 PCs

For a gaming café with 10 PCs, I would choose CAT6 Ethernet cables and an RJ45 crimping tool. CAT6 supports 1 Gbps Ethernet over standard channels up to 100 metres, which is suitable for most small cafés and provides stable, low-latency connections. Ethernet cables and RJ45 connectors are generally cheaper and easier to install than fiber optic equipment. A fiber cleaver is unnecessary because I am using copper Ethernet cables rather than fiber optics.